A series composite material pump

Specially designed to move hazardous and highly corrosive liquid. Especially applicable to various hardening agents, acids, alkalis, chlorides and other chemicals.

Advantage

- extremely corrosion resistant

- Designated materials to maximize life span

- Specially designed structure for reliable sealing

- Simple design with only two moving parts

- Easy maintenance and service with no need for special tools

Material: reinforced carbon composite material, high chromium-nickel alloy content

Stealing: single and double mechanical seal

Installation: motor and footing installations

Pressure: 1-10 bar (depending upon viscosity)

Viscosity: up to 5000cps

Temperature: -5-60℃

Typical application

Water treatment industry:PAC, PAM, hydrochloric acid, phosphoric acid, sodium hydrogen sulfife, and hypochlorous acid etc

Organic chemical industry: acetic acid, oxalic acid, ammonium hydroxide, lipid and amine

Casting industry: 4-toluene sulfonic acid

Performance parameter table

| type | Single rotation displacement(cc/rev) | Rated flow rate at maximum speed(L/min) | Maximum speed (rpm) | Inlet/outlet dimension |
| --- | --- | --- | --- | --- |
| AS190 | 1.90 | 2.5 | 1500 | G1/2” |
| AS285 | 2.85 | 4.2 | 1500 | G1/2” |
| AS380 | 3.80 | 5.7 | 1500 | G1/2” |
| AS570 | 5.7 | 8.5 | 1500 | G1/2” |
| AS855 | 8.55 | 12.8 | 1500 | G1/2” |
| AM320 | 32.0 | 48.0 | 1500 | G3/4 |

Note: The above parameters are measured when the pressure is 0 bar